# Discussion notes

Noise Model

- Gaussian noise is applied at the start. This is implemented to account for a variety of physical sources of noise, e.g. thermal + amplifier + EMI. This means it can be moved around but will affect results of the applied noise model due to non-commutativity of multiplicative and additive noise types.

- If Gaussian is mainly front-end electronics noise, a more literal chain is often:

  - contact/bias → Gaussian electronics noise → quantisation.

- Current implementation puts Gaussian first, which effectively let’s contact impedance also scale that Gaussian draw. That is not wrong, but it implies Gaussian includes pre-interface disturbance too.

- Quantisation-last is unambiguously physical as ADC error should be at the end. Need references for ADC error.

- More work needs to be done to split gaussian noise up into a more physical quantity and describe what is contributing to it in the real world. This will provide better insight into where in the chain this should go.

- Choice was made to keep gaussian first as it builds on previous studies that augment data with gaussian only, essentially building on the previous noise pipelines and not changing them.

- Abolition study investigates order of gaussian purely to show difference in results, but any changes should be backed up by physical qualities.

EDA

- No duplicates except for no touch class as expected

- Scalling appears to have little effect on distribution.

- Dimensionality reduction harms performance based on higher f1/acc mean and lower f1/acc std. The 1D-CNN exploits spatial adjacency patterns in the electrode array. Removing features destroys this structure.

- RobustScaler outperforms Standard Scalar and consistent across all 15 folds. Results are negligible.

- Check if results confirm the above.